Step 1.6 — Non-Dimensionalisation of the Coupled Hydro-Mechanical PDEs

Işıkdere Lignite Open-Pit Mine · PINN Slope-Stability Framework · Phase 1

# 1. Purpose and Rationale

The Richards equation and the Navier–Cauchy equilibrium equations span roughly twelve orders of magnitude in their raw SI form: saturated conductivity is K_s ≈ 10⁻⁶ m/s while effective stresses are σ ≈ 10⁶ Pa. Feeding variables of such disparate magnitude into a network of tanh neurons drives the activations into their flat, saturated regions, the gradients vanish, and training stalls at a near-random state. Non-dimensionalisation rescales every coordinate, field, and residual so that inputs, outputs, and PDE residual terms live near the interval [−1, 1]. It is the precondition that lets the adaptive loss weighting (GradNorm) start from a meaningful balance rather than spending its warm-up budget fighting a fixed 10⁶ disparity between loss terms.

# 2. Characteristic Scales

The reference values are chosen from the Işıkdere geometry so that the resulting dimensionless groups keep a clear physical meaning. Arbitrary scales would work numerically but would produce residual coefficients that no reviewer could interpret.

| Symbol | Value (SI) | Meaning / justification |
| --- | --- | --- |
| L_ref | 170 m | Pit depth — characteristic spatial length |
| T_ref | 86 400 s (1 day) | Transient seepage timescale; matches drawdown resolution |
| H_ref | 30 m | Maximum piezometric head above the coal seam |
| σ_ref | 1 × 10⁶ Pa (1 MPa) | Typical overburden stress |
| K_ref | 1 × 10⁻⁶ m/s | Representative saturated hydraulic conductivity |
| E_ref | 1 × 10⁹ Pa (1 GPa) | Representative rock-mass modulus |
| U_ref | 0.17 m | Displacement scale, U_ref = σ_ref·L_ref / E_ref |

## 2.1 Dimensionless variables

x* = x / L_ref     z* = z / L_ref     t* = t / T_ref

ψ* = ψ / H_ref     u* = u / U_ref     σ* = σ / σ_ref

With these choices x* ∈ [0, 1] horizontally, z* ∈ [0, ~4.4] across the 750 m width, t* is measured in days, and ψ* ∈ [−1, +1] across the suction/pressure range.

# 3. Dimensionless Governing Equations

## 3.1 Richards equation (mixed form)

Substituting the scaled variables into the mixed-form Richards equation and dividing through by the storage term gives:

C*(ψ*)·∂ψ*/∂t*  =  Π_R,diff · ∂/∂x*[ K*(ψ*) ∂ψ*/∂x* ]  +  Π_R,grav · ∂K*/∂z*

where the dimensionless groups are Π_R,diff = T_ref·K_ref / L_ref² and Π_R,grav = K_ref·T_ref / L_ref. The head-vs-length ratio Π_R,hz = H_ref / L_ref appears wherever pressure head and elevation are combined as (ψ + z), because the two are scaled differently.

## 3.2 Mechanical equilibrium (Navier–Cauchy)

Dividing the equilibrium equation by σ_ref / L_ref yields:

∇*·σ*_eff  +  Π_M,body · (ρ_b / ρ_b,ref) · e_z  =  0

with body-force group Π_M,body = ρ_b,ref·g·L_ref / σ_ref. The Bishop effective stress that couples seepage into the mechanical residual becomes:

σ*_eff = σ* − Π_M,couple · χ · ψ*,     Π_M,couple = ρ_w·g·H_ref / σ_ref

# 4. Numerical Values of the Dimensionless Groups

| Group | Expression | Value | Magnitude |
| --- | --- | --- | --- |
| Π_R,diff | T·K / L² | 2.99 × 10⁻⁶ | far below O(1) |
| Π_R,grav | K·T / L | 5.08 × 10⁻⁴ | below O(1) |
| Π_R,hz | H / L | 1.76 × 10⁻¹ | O(1) |
| Π_M,body | ρ·g·L / σ | 3.00 | O(1) |
| Π_M,couple | ρ_w·g·H / σ | 2.94 × 10⁻¹ | O(1) |

# 5. Important Caveat — the Seepage Term Is Not Automatically O(1)

The mechanical residual is well-balanced: its three groups are all O(1). The Richards residual is not. With a one-day reference time, the diffusion group is Π_R,diff ≈ 3 × 10⁻⁶ — the same six-order disparity that motivated the exercise, now relocated from the variables into the coefficient. This is physically correct: the diffusive timescale of the pit is

T_diff = L_ref² / K_ref = 170² / 10⁻⁶ ≈ 2.9 × 10¹⁰ s ≈ 916 years

so on a one-day residual the seepage term genuinely should be tiny — a pressure signal barely moves across the pit in a day. Non-dimensionalising the variables alone therefore does not, by itself, place every residual term at O(1); the modeller must still address the seepage coefficient. Two defensible routes:

- Keep T_ref = 1 day (to resolve transient rainfall response) and let the adaptive weighting or an explicit per-term normalisation absorb Π_R,diff, so the seepage residual is trained at its own scale rather than swamped.

- Adopt the diffusive timescale T_ref = L²/K when the target is quasi-steady drawdown; then Π_R,diff = 1 exactly and the gravity group Π_R,grav = L/H·(H/L)-type term rescales accordingly.

Recommendation for this thesis: retain T_ref = 1 day because the failures at Işıkdere followed a heavy-rainfall week and the transient response is the phenomenon of interest, and pair it with per-residual normalisation inside the loss (report Π_R,diff explicitly so the weighting is reproducible). State this choice openly in the methods section — reviewers of PINN papers specifically look for it.

# 6. Consequence for the Loss Function

Once the variables are non-dimensionalised and the seepage coefficient is handled as above, every residual enters the multi-objective loss at a comparable magnitude, and GradNorm has a meaningful starting point:

L(θ) = w_PDE·L_PDE + w_BC·L_BC + w_IC·L_IC + w_data·L_data

If this step is skipped, the adaptive weighting spends its entire warm-up trying to rebalance loss terms that differ by 10⁶, which typically diverges before it converges.

# 7. Reproducible Implementation

All scales and dimensionless residuals are centralised in nondim.py and verified by validate_nondim.py, which performs an algebraic raw→dimensionless→raw round-trip (error < 10⁻¹²) and reports the magnitude of every group. Import the module everywhere so the network, the collocation sampler, and the residual functions share one consistent scaling.

from nondim import SCALES, richards_residual_nd, mechanical_residual_nd

      
# dimensionless coordinates feed the network; autograd differentiates on them
r_R = richards_residual_nd(C_star, dpsi_dt_star, div_Kgrad_star, dK_dz_star)
r_Mx, r_Mz = mechanical_residual_nd(div_sigma_eff_star, rho_ratio)
L_PDE = (r_R**2).mean() + (r_Mx**2 + r_Mz**2).mean()

Output → feeds Phase 2: dimensionless governing equations with the mechanical residual at O(1) and the seepage residual explicitly scaled and documented. Phase 1 is complete.