PHASE 1 — REFERENCE STATE (COMPLETED)

PINN-HM Coupled Hydro-Mechanical Slope Stability Analysis

Işıkdere Lignite Open-Pit Mine · Milas Basin, Muğla, Turkey

Primary source: Ulusay, R., Ekmekci, M., Tuncay, E. & Hasancebi, N. (2014). Engineering Geology, 181, 261–280. DOI: 10.1016/j.enggeo.2014.08.005.

This document consolidates the Phase-1 reference state assembled in Steps 1.1–1.5 of the project roadmap. Every derived quantity below is traceable to a reported field/lab value plus a named published correlation. Values are given in SI unless noted. The final section is the Phase-1 completion checklist confirming that K_s, ρ_dry, ρ_sat, E, ν, (α, n, θ_s, θ_r), (σ_ci, m_b, s, a), and n₀ can now be stated for every stratum.

# 1. Master Reference-State Table

## 1a. Hydraulic, density & elastic properties

| Stratum | K_s (m/s) | ρ_dry (kg/m³) | ρ_sat (kg/m³) | E (MPa) | ν |
| --- | --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 1.00e-9 | 1519 | 1519 | 478 | 0.28 |
| Weak-zone marl (Sekköy Fm.) | 1.00e-9 | 1723 | 1723 | 87 | 0.3 |
| Coal seam (lignite) | 2.90e-6 | 877 | 877 | 94 | 0.32 |
| Tuffite (Yatağan Fm.) | 1.00e-9 | 1468 | 1468 | 32 | 0.33 |
| Underclay (Turgut Fm.) | 1.00e-9 | 1543 | 1876 | soil | 0.4 |
| Slope debris | n/a | 1347 | 1794 | soil | 0.35 |

K_s floor value 1×10⁻⁹ m/s assigned to aquitards (marl, underclay) for numerical stability; coal uses the 0.1–0.5 m/day range midpoint. Densities from ρ = γ × 101.94; underclay/slope-debris ρ_dry from γ_dry = γ_sat/(1+w). E via Hoek–Diederichs (2006); soil layers carry no rock-mass modulus.

## 1b. van Genuchten SWCC parameters

| Stratum | α (1/m) | n (–) | θ_s (m³/m³) | θ_r (m³/m³) | m = 1−1/n |
| --- | --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 0.5 | 1.2 | 0.38 | 0.05 | 0.167 |
| Weak-zone marl (Sekköy Fm.) | 0.5 | 1.2 | 0.38 | 0.05 | 0.167 |
| Coal seam (lignite) | 2 | 1.8 | 0.15 | 0.02 | 0.444 |
| Tuffite (Yatağan Fm.) | 0.5 | 1.2 | 0.38 | 0.05 | 0.167 |
| Underclay (Turgut Fm.) | 1 | 1.23 | 0.43 | 0.07 | 0.187 |
| Slope debris | 0.8 | 1.15 | 0.48 | 0.09 | 0.130 |

Soil layers (underclay, slope debris) from Rosetta3 pedotransfer (Zhang & Schaap, 2017) using reported grain-size + bulk density. Marl family from published soft-calcareous-marl values; coal from an equivalent-continuum single-porosity fit.

## 1c. Hoek–Brown parameters & initial porosity

| Stratum | σ_ci (Pa) | m_i | GSI | D | m_b | s | a | n₀ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 1.79e+7 | 4 | 50 | 1 | 0.113 | 2.40e-4 | 0.506 | 0.427 |
| Weak-zone marl (Sekköy Fm.) | 4.29e+6 | 3 | 45 | 1 | 0.059 | 1.04e-4 | 0.508 | 0.427 |
| Coal seam (lignite) | 7.80e+6 | 18 | 40 | 1 | 0.248 | 4.54e-5 | 0.511 | 0.374 |
| Tuffite (Yatağan Fm.) | 3.59e+6 | 15 | 26 | 1 | 0.076 | 4.40e-6 | 0.529 | 0.4 |
| Underclay (Turgut Fm.) | soil | — | — | — | — | — | — | 0.383 |
| Slope debris | soil | — | — | — | — | — | — | 0.492 |

Hoek–Brown 2002 reduction: m_b = m_i·exp((GSI−100)/(28−14D)); s = exp((GSI−100)/(9−3D)); a = 0.5 + (1/6)[exp(−GSI/15) − exp(−20/3)], with D = 1 (production blasting). n₀ = 1 − γ_dry/(γ_w·G_s), γ_w = 9.81 kN/m³. Coal bulk n₀ = 0.374 is correct for the elastic-compressibility term; its fracture porosity (~0.15) governs advective transport if a dual-porosity treatment is later adopted.

# 2. Derivation Methods (traceability)

## 2.1 Initial porosity n₀ — Kozeny–Carman reference state

Identity: n₀ = 1 − γ_dry/(γ_w·G_s), with γ_dry = γ_sat/(1+w). Worked results: underclay n₀ = 0.383 (γ_dry 15.14, G_s 2.50); marl 0.427 (γ 14.9, G_s 2.65); coal 0.374 (γ 8.6, G_s 1.40, lignite); slope debris 0.492 (γ_dry 13.21, G_s 2.65). Inside the PINN, porosity updates as n = n₀ + ε_v with ε_v = ∂u/∂x + ∂v/∂z, feeding the Kozeny–Carman update K_s = K_s0·(n/n₀)³·((1−n₀)/(1−n))² on the same collocation points — closing the two-way loop without outer iteration.

## 2.2 Elastic modulus E_rm — Hoek–Diederichs (2006)

E_rm = E_i·[0.02 + (1−D/2)/(1+exp((60+15D−GSI)/11))], E_i = MR·UCS. Modulus ratios: marl 400, coal 300, tuffite 350. Results: marl ≈478 MPa, weak-zone marl ≈87 MPa, coal ≈94 MPa, tuffite ≈32 MPa. The intact modulus E_i is deliberately degraded — the slope behaves as a jointed rock mass, not a lab specimen.

## 2.3 Poisson's ratio ν

Literature consensus by lithology: marl 0.28, weak-zone marl 0.30, coal 0.32, tuffite 0.33, underclay ~0.40, slope debris ~0.35. ν enters the earth-pressure coefficient K₀ = ν/(1−ν) used in the Step 2.3 initial condition, so it is not cosmetic.

## 2.4 van Genuchten SWCC — Rosetta3 / literature

Underclay: θ_r 0.07, θ_s 0.43, α 1.0 /m, n 1.23 (Rosetta3 from sand 22.5 / silt 46 / clay 31.5, ρ 1876). Slope debris: θ_r 0.09, θ_s 0.48, α 0.8 /m, n 1.15. Marl family: θ_r 0.05, θ_s 0.38, α 0.5 /m, n 1.20. Coal: θ_r 0.02, θ_s 0.15, α 2.0 /m, n 1.80. The parameter n is the most sensitive input; carry the adopted values into the Step 6.2 sensitivity analysis.

## 2.5 Hoek–Brown m_b, s, a — Hoek–Brown (2002)

Adopted intact-rock m_i (RocScience tables): marl 4, weak-zone marl 3, coal 18, tuffite 15. Results: marl (m_b 0.113, s 2.40×10⁻⁴, a 0.506); weak-zone marl (0.059, 1.04×10⁻⁴, 0.508); coal (0.248, 4.54×10⁻⁵, 0.511); tuffite (0.076, 4.40×10⁻⁶, 0.529). These feed the GHB-SSR reduction of Step 5.2 alongside the MC-SSR sweep of Step 5.1.

# 3. Phase-1 Completion Checklist

Phase 1 is complete when, for every stratum in the chosen cross-section, all of the following can be stated. ✔ = value assembled above.

✔  K_s (saturated hydraulic conductivity, m/s) — all strata

✔  ρ_dry and ρ_sat (dry & saturated density, kg/m³) — all strata

✔  E (rock-mass elastic modulus) — 4 rock strata; soil layers flagged as soil

✔  ν (Poisson's ratio) — all strata

✔  (α, n, θ_s, θ_r) van Genuchten SWCC — all strata (+ m = 1−1/n)

✔  (σ_ci, m_b, s, a) Hoek–Brown — 4 rock strata (soil layers use MC only)

✔  n₀ (initial porosity) — all strata; Kozeny–Carman reference state assembled

Open items to refine before Phase 2 (not blocking): tuffite n₀ is an estimate (no water content reported) — refine if a measured value surfaces; coal advective transport may warrant a dual-porosity fracture value (~0.15); van Genuchten sets for the marl family and tuffite are literature/proxy rather than site-specific, and rainfall-intensity Neumann BC data (MGM Muğla station) is still to be acquired for Phase 2.

Next: Step 1.6 non-dimensionalisation (reference scales L_ref = pit depth 170 m, H_ref = piezometric head above coal, σ_ref = 1 MPa, T_ref = 1 day), after which Phase 1 closes and Phase 2 (geometric domain) begins.