Extracted Numerical Data for PINN Framework

from the Işıkdere Lignite Open-Pit Mine

Primary Source: Ulusay, R., Ekmekci, M., Tuncay, E., & Hasancebi, N. (2014). Engineering Geology, 181, 261–280.

DOI: 10.1016/j.enggeo.2014.08.005

Mine: Işıkdere Lignite Open Pit, Milas Basin, Muğla Province, Western Turkey

Operated by: Turkish Coal Enterprises (TKI) since 2009

Data includes: 423 Packer tests | 64 unit weight tests | 15 shear test sets | 452 structural measurements | 30 boreholes | 23 monitoring wells

# Table 1: Physical & Index Properties

Extracted from Table 2 of the paper. Values in parentheses show (mean/number of specimens tested). These provide γ (unit weight) for body forces (ρ_b·g) and grain size data for deriving van Genuchten SWCC parameters via pedotransfer functions.

| Material | γ (kN/m³) | w (%) | G_s | Sand (%) | Silt (%) | Clay (%) | LL | PL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FROM TABLE 2 — Physical and Index Properties (Ulusay et al., 2014) |  |  |  |  |  |  |  |  |
| Marl (Sekköy Fm.) | 14.8–15.0 (mean 14.9, n=16) | — | — | — | — | — | — | — |
| Weak zone-marl (Sekköy Fm.) | 16.0–17.8 (mean 16.9, n=10) | — | — | — | — | — | — | — |
| Tuffite (Yatağan Fm.) | 13.8–15.3 (mean 14.4, n=12) | — | — | — | — | — | — | — |
| Coal | 8.4–8.7 (mean 8.6, n=11) | — | — | — | — | — | — | — |
| Underclay | 16.1–20.5 (mean 18.4, n=3) | 17.8–25.8 (mean 21.5, n=9) | 2.44–25.7 (mean 25.0, n=6) | 12–33 (mean 22.5) | 32–60 (mean 46) | 28–35 (mean 31.5) | 46.6–55.6 (mean 51.1) | 20.3–25.5 (mean 22.9) |
| Slope debris | 16.7–18.1 (mean 17.6, n=6) | 19.3–40.6 (mean 33.2, n=6) | — | 3–4 (mean 3.5) | 53–54 (mean 53.5) | 42–44 (mean 43) | 61.8–63.0 (mean 62.4) | 28.1–28.7 (mean 28.4) |
| Fault gouge | — | — | — | 20 | 80 | — | 61.9 | NP |

# Table 2: Shear Strength Parameters

Extracted from Table 3a and back-analysis results (Sections 7, 8). The back-analysis validated c = 4.4 kPa, φ = 15.4° (residual, smooth bedding) as the design parameters. For your PINN: these are the c′ and φ′ values to reduce via SRF in the Mohr-Coulomb SSR workflow.

| Sample Type | Test Type | No. Specimens | Peak c (kPa) | Peak φ (°) | Residual c (kPa) | Residual φ (°) | PINN Usage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FROM TABLE 3a — Shear Strength of Discontinuities & Soils (Direct Shear Box Tests) |  |  |  |  |  |  |  |
| Bedding — without infill (marl) | Water-filled shear box | 8 | 0.0 | 27.9 | 0.0 | 14.4 | Mohr-Coulomb SSR |
| Bedding — without infill (marl) | Lubricated surface | 8 | 5.4 | 28.0 | 4.4 | 15.4 | Mohr-Coulomb SSR |
| Bedding — slickensided with infill (marl) | Lubricated surface | 7 | 0.9 | 14.0 | 0.0 | 12.5 | Mohr-Coulomb SSR |
| Bedding (tuffite) | Lubricated surface | 9 | 0.0 | 34.2 | 0.0 | 32.9 | Mohr-Coulomb SSR |
| Underclay | Consolidated-drained (CD) | 9 | 10.1 | 19.2 | 13.6 | 13.1 | Mohr-Coulomb SSR |
| Fault gouge | Consolidated-drained (CD) | 4 | 16.3 | 29.4 | 14.0 | 19.1 | Mohr-Coulomb SSR |
| Slope debris | Consolidated-drained (CD) | 6 | 8.9 | 21.7 | 8.0 | 15.8 | Mohr-Coulomb SSR |
| BACK-ANALYSIS VALIDATED PARAMETERS (for design use) |  |  |  |  |  |  |  |
| Slickensided bedding (INS-1) | Back-analysis at F=1 | — | ~1.0 | ~15.0 | — | — | Validated by back-analysis |
| Smooth bedding (INS-2 & 3) | Back-analysis at F=1 | — | 4.4 | 15.4 | — | — | Recommended for design: residual strength of smooth bedding free from infill |

# Table 3: Hoek-Brown Failure Criterion Parameters

Extracted from Table 3b. D = 1 for all units (production blasting in large open pit). *GSI values for coal and tuffite derived from RMR. Use these directly in the Generalized Hoek-Brown SSR: reduce m_b, s, a by SRF to find FOS.

| Rock Type | GSI | UCS (MPa) | m_i | D | UCS Source | GSI Source | PINN Usage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FROM TABLE 3b — Hoek-Brown Failure Criterion Input Parameters |  |  |  |  |  |  |  |
| Sekköy Fm. (marl/marl-limestone) | 50 | 17.9 | 4 | 1 | Uniaxial compression test (ISRM 2007) | Quantitative GSI chart (Sonmez & Ulusay, 2002) | GHB SSR: derive m_b, s, a |
| Sekköy Fm. (weak zone — northern slope) | 45 | 4.29 | 3 | 1 | Uniaxial compression test | Quantitative GSI chart | GHB SSR: weak zone |
| Coal seam | 40* | 7.80 | 18 | 1 | From Pasamehmetoglu et al. (1988), adjacent pit | Estimated from RMR: GSI = RMR₉₉ − 5 | GHB SSR: coal layer |
| Tuffite (Yatağan Fm.) | 26* | 3.59 | 15 | 1 | Block Punch Index (BPI) test, ISRM 2007 | Estimated from RMR: GSI = RMR₉₉ − 5 | GHB SSR: tuffite layer |

# Table 4: Hydraulic Properties & Groundwater Data

Extracted from Sections 5.1–5.5. The 423 Packer (Lugeon) tests provide K values. The coal seam is the critical confined aquifer affecting slope stability. Convert K to m/s for Richards equation: 0.1 m/day = 1.16×10⁻⁶ m/s. The transient analytical solution (Nguyen & Raudkivi, 1982) provides piezometric drawdown predictions usable as PINN validation data.

| Hydrostratigraphic Unit | Permeability (Lugeon) | K (m/day) | Hydraulic Head (m asl) | Storage Coeff. | PINN Usage |
| --- | --- | --- | --- | --- | --- |
| FROM SECTION 5.2 — Hydraulic Characterization (423 Packer/Lugeon Tests) |  |  |  |  |  |
| Karstic bedrock carbonate aquifer | Up to 25 Lu | ~0.27 | 113–197 (confined) | — | Lower BC (head below pit bottom) |
| Coal seam aquifer (confined) | ~5 Lu | 0.1–0.5 | >205 (confined, variable) | 0.00003 (Lohman, 1972) | Richards eq. K_s; L_IC (piezometric head) |
| Lacustrine limestone (unconfined) | 5–70 Lu | 0.056–0.86 | Sat. thickness ~5 m | — | Upper aquifer, limited extent |
| Medium-thick bedded marl (Sekköy Fm.) | ~0 Lu (no water intake) | Essentially impervious | — | — | Confining layer (aquitard/aquiclude) |
| Underclay (Turgut Fm.) | Impervious | Effectively zero | — | — | Confining layer separating coal from bedrock |
| Alluvium (western boundary) | — | — | Near surface (unconfined) | — | Surface BC at stream boundary |
| TRANSIENT FLOW PREDICTIONS (Nguyen & Raudkivi, 1982 analytical solution) |  |  |  |  |  |
| Coal seam inflow rate (initial) | — | — | — | — | 0.056 m³/day per m at start; decays exponentially |
| Coal seam total inflow (30 days) | — | — | — | — | 5.37 m³ per m of excavation length |
| Drawdown at excavation face | — | — | — | — | ~20 m decline in ~1 week at excavated surface |
| Temporal stability (C_v) | — | — | — | — | C_v < 5% in most wells (< 10% in all); indirect recharge |

# Table 5: Structural, Geometric & Stability Data

Compiled from Sections 2–4, Table 1, and Sections 8.2. The FOS values from limit equilibrium analyses serve as benchmark validation targets for your PINN-SSR predictions. The structural data defines discontinuity sets for domain decomposition.

| Parameter | Value from Paper | PINN Usage |
| --- | --- | --- |
| PIT GEOMETRY |  |  |
| Pit dimensions (planned) | Length: 2.5 km, Width: 750 m, Max depth: 170 m | Computational domain boundaries |
| Overall final slope angle (initial design) | 30°–32° (northern boundary) | Slope geometry for SSR analysis |
| Bench geometry | 7–10 m high benches, 10–12 m wide berms, 60° face angle | Bench-scale boundary conditions |
| Coal seam thickness | 1–30 m (max ~30 m, thins toward basin margins) | Domain layer thickness |
| Overburden thickness (Sekköy Fm.) | 50–230 m | Overburden domain thickness |
| Slope debris thickness | 2–12 m (max ~10 m in north) | Surface layer definition |
| Alluvium thickness | 2–8.5 m (western boundary) | Stream boundary layer |
| Redesigned slope angles (northern wall) | 22°–24° (sections 4,5,7,9); 30° (sections 3,8,16) | Validation target FOS ≥ 1.3 |
| STRUCTURAL DISCONTINUITIES (from 452 measurements, 19 scan-line locations) |  |  |
| Bedding dip range | 2°–26° (increases toward basin margins N and S) | Planar failure geometry; domain decomposition interfaces |
| Joint set J1 | Dip/dip direction: 84°/171° | Discontinuity in PINN domain |
| Joint set J2 | Dip/dip direction: 86°/012° | Discontinuity in PINN domain |
| Joint set J3 | Dip/dip direction: 75°/071° | Discontinuity in PINN domain |
| Joint spacing (J1–J3) | 0.64–1.02 m; infill: none/thin coating; Jv = 10.64 | Rock mass characterization |
| Tuffite joint spacing | 7–20 cm; bedding spacing < 5 cm | Tuffite domain properties |
| Coal seam bedding spacing | 5–30 cm; joint spacing 10–30 cm; RQD ~70% | Coal domain properties |
| Fault zone width | 50–300 mm | Fault conduit modeling in Richards equation |
| Fault dip / throw | 65°–75° / 1–20 m | Structural BC; domain discontinuities |
| Main basin-margin faults | F1 (north), F17 (south): NW–SE trending, high persistence | Critical for multi-planar failure; defines aquifer compartments |
| STABILITY ANALYSIS RESULTS (FOS values for validation) |  |  |
| Section 1 — initial project (30°) | F = 1.24 (compound failure) | Validation benchmark for PINN-SSR |
| Section 2 — initial project (30°) | F = 0.82 (compound failure) | Validation benchmark |
| Section 4 — initial project (30°) | F = 1.01 (compound failure) | Validation benchmark |
| Section 5 — initial project (30°) | F = 0.94 (non-circular failure) | Validation benchmark |
| Section 7 — with coal seam excavated | F = 1.19 (compound through coal + underclay) | Coupled HM validation (pore pressure effect) |
| Section 1 south — initial (30°) | F = 0.74 (multi-planar failure) | Most critical; validation target |
| Section 1 south — shifted + 30° | F = 1.32 (with pore water pressure) | Remediated design validation |
| Design FOS criterion | F ≥ 1.3–1.4 (final walls); F ≥ 1.1 (benches) | Target FOS for PINN-SSR convergence |

# Table 6: Gap Analysis — Missing Parameters & Derivation Strategy

Parameters required by your PINN framework but not directly reported in the paper. Each entry includes a derivation strategy using either the paper’s existing data or established correlations from published literature.

| Missing Parameter | Why You Need It | How to Derive / Supplement | Priority |
| --- | --- | --- | --- |
| PARAMETERS NOT IN PAPER — Required for Your PINN Framework |  |  |  |
| van Genuchten α, n, θ_s, θ_r (SWCC) | Richards equation constitutive model | Use Rosetta pedotransfer model with grain size + Atterberg limits from Table 2 (underclay, slope debris); for marl: use published correlations for calcareous clay | Essential |
| Young’s modulus (E) | Hooke’s law — stress-strain in mechanical PDE | Derive from GSI + UCS via Hoek-Brown: E_rm = E_i · (0.02 + (1−D/2)/(1+exp((60+15D−GSI)/11))); or use published values for marl (E ~ 0.5–5 GPa), coal (E ~ 1–3 GPa) | Essential |
| Poisson’s ratio (ν) | Elasticity tensor D | Published correlations: marl ν ~ 0.25–0.30; coal ν ~ 0.30–0.35; underclay ν ~ 0.35–0.45 | Essential |
| K_s in m/s (SI units) | Richards equation in SI | Convert from paper: coal K = 0.1–0.5 m/day = 1.16×10⁻⁶ to 5.79×10⁻⁶ m/s; marl: effectively 0 | Essential |
| Porosity (n₀) per stratum | Kozeny-Carman feedback; initial condition | Estimate from unit weight: n = 1 − (γ_d/(γ_w · G_s)); underclay G_s = 2.50, γ given | Essential |
| Rainfall intensity data | Surface flux Neumann BC for Richards eq. | Obtain from Turkish State Meteorological Service (MGM) for Muğla station; INS-2 & 3 occurred after heavy rainfall week | Essential |
| Piezometer time-series (raw data) | L_data validation; transient calibration | Paper provides only statistics (min/max/mean, C_v < 5%); contact authors or use analytical predictions from Fig. 10 | Important |
| Biot’s coefficient (α_B) | Poroelastic storage for two-way coupling | For marl/coal: α_B ≈ 1.0 (common assumption for soft rocks); or derive from K_solid | Recommended |

Summary: This document extracts all quantitative data from Ulusay et al. (2014) organized into 6 tables: physical properties (7 materials), shear strength (7 sample types + 2 back-analysis results), Hoek-Brown parameters (4 rock types), hydraulic properties (6 units + 4 transient predictions), structural/geometric data (8 geometry + 10 structural + 8 FOS values), and a gap analysis of 8 missing parameters with derivation strategies. Total: ~80 individual data points directly usable for PINN implementation.