# Step 1.1 — Unit Conversion to SI

Master Parameter Sheet for the Işıkdere Lignite Open-Pit Mine (Primary Source: Ulusay et al., 2014, Engineering Geology)

Purpose: Convert every parameter reported by Ulusay et al. (2014) to strict SI units — metres (m), seconds (s), Pascals (Pa), and kilograms per cubic metre (kg/m³) — so that the Richards equation, the mechanical equilibrium equation, and all constitutive models in the PINN framework are evaluated in a single consistent unit system. Each row lists the original published value, the exact conversion factor applied, and the resulting SI value, side by side, per stratum. Mean values are shown with the reported ranges in parentheses where the paper gives ranges.

## Conversion Conventions

Gravitational acceleration: g = 9.81 m/s² (consistent with the project roadmap; using g = 9.80665 m/s² changes densities by < 0.04%).

## Table 0 — Conversion Factors Applied

| Quantity | Original unit | SI target unit | Conversion factor / formula |
| --- | --- | --- | --- |
| Hydraulic conductivity, K | m/day | m/s | ÷ 86 400 (× 1.1574 × 10⁻⁵) |
| Permeability (field) | Lugeon (Lu) | m/s | 1 Lu ≈ 1.3 × 10⁻⁷ m/s (cross-check only; paper reports m/day) |
| Unit weight, γ | kN/m³ | kg/m³ (density ρ) | ρ = γ × 1000 / 9.81 (× 101.94) |
| Unit weight, γ (body force) | kN/m³ | N/m³ | × 1000 |
| Cohesion, c | kPa | Pa | × 10³ |
| Stress / UCS, σ_ci | MPa | Pa | × 10⁶ |
| Friction angle, φ | degrees (°) | radians (rad) | × π/180 (× 0.0174533); tan φ also tabulated (dimensionless) |
| Water content, w | % | decimal (–) | ÷ 100 |
| Time | day | s | × 86 400 |
| Inflow rate | m³/day per m | m³/s per m | ÷ 86 400 |
| Length / head | m | m | × 1 (already SI) |
| GSI, mᵢ, D, Gₛ, storage coeff. S | dimensionless | dimensionless | no conversion |

# Table A — Physical & Index Properties per Stratum

Source: Ulusay et al. (2014), Table 2. Values are mean (range / no. of specimens as reported).

| Stratum / material | Original value | Orig. unit | Conversion factor | SI unit | SI value |
| --- | --- | --- | --- | --- | --- |
| A.1 Bulk unit weight γ (kN/m³) → bulk density ρ (kg/m³) [Paper Table 2; ρ = γ × 101.94] |  |  |  |  |  |
| Marl (Sekköy Fm.) | γ = 14.9 (14.8–15.0) | kN/m³ | × 101.94 | kg/m³ | ρ = 1 519 (1 509 – 1 529) |
| Weak-zone marl (Sekköy Fm.) | γ = 16.9 (16.0–17.8) | kN/m³ | × 101.94 | kg/m³ | ρ = 1 723 (1 631 – 1 815) |
| Tuffite (Yatağan Fm.) | γ = 14.4 (13.8–15.3) | kN/m³ | × 101.94 | kg/m³ | ρ = 1 468 (1 407 – 1 560) |
| AEWWWWWCoal seam | γ = 8.6 (8.4–8.7) | kN/m³ | × 101.94 | kg/m³ | ρ = 877 (856 – 887) |
| Underclay (Turgut Fm.) | γ = 18.4 (16.1–20.5) | kN/m³ | × 101.94 | kg/m³ | ρ = 1 876 (1 641 – 2 090) |
| Slope debris | γ = 17.6 (16.7–18.1) | kN/m³ | × 101.94 | kg/m³ | ρ = 1 794 (1 702 – 1 845) |
| Fault gouge | not reported | — | — | — | not reported in paper |
| A.2 Body-force form of unit weight γ (kN/m³ → N/m³, used in ρ·g source term) |  |  |  |  |  |
| Marl (Sekköy Fm.) | 14.9 | kN/m³ | × 10³ | N/m³ | 1.49 × 10⁴ |
| Weak-zone marl | 16.9 | kN/m³ | × 10³ | N/m³ | 1.69 × 10⁴ |
| Tuffite | 14.4 | kN/m³ | × 10³ | N/m³ | 1.44 × 10⁴ |
| Coal seam | 8.6 | kN/m³ | × 10³ | N/m³ | 8.6 × 10³ |
| Underclay | 18.4 | kN/m³ | × 10³ | N/m³ | 1.84 × 10⁴ |
| Slope debris | 17.6 | kN/m³ | × 10³ | N/m³ | 1.76 × 10⁴ |
| A.3 Index properties (dimensionless targets) |  |  |  |  |  |
| Underclay — water content w | 21.5 % (17.8–25.8 %) | % | ÷ 100 | – (decimal) | 0.215 (0.178 – 0.258) |
| Slope debris — water content w | 33.2 % (19.3–40.6 %) | % | ÷ 100 | – (decimal) | 0.332 (0.193 – 0.406) |
| Underclay — specific gravity Gₛ | 2.50 (mean of reported set) | – | none | – | 2.50 (already dimensionless) |

# Table B — Shear Strength Parameters per Stratum / Discontinuity

Source: Ulusay et al. (2014), Table 3a and Section 7. c converted kPa → Pa; φ given in degrees, radians, and as tan φ.

| Sample type | Original c | Original φ | Conversion | SI cohesion c | φ (rad) | tan φ (–) | PINN usage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B.1 PEAK shear strength (direct shear box; Paper Table 3a) c: kPa → Pa (× 10³); φ: ° → rad (× 0.0174533), tan φ dimensionless |  |  |  |  |  |  |  |
| Bedding, no infill — marl (water-filled box) | c = 0.0 kPa | φ = 27.9° | × 10³ ; × π/180 | c = 0 Pa | φ = 0.4869 rad | tan φ = 0.5295 | MC yield, SSR sweep |
| Bedding, no infill — marl (lubricated) | c = 5.4 kPa | φ = 28.0° | × 10³ ; × π/180 | c = 5.40 × 10³ Pa | φ = 0.4887 rad | tan φ = 0.5317 | MC yield, SSR sweep |
| Bedding, slickensided w/ infill — marl | c = 0.9 kPa | φ = 14.0° | × 10³ ; × π/180 | c = 9.0 × 10² Pa | φ = 0.2443 rad | tan φ = 0.2493 | MC yield, SSR sweep |
| Bedding — tuffite | c = 0.0 kPa | φ = 34.2° | × 10³ ; × π/180 | c = 0 Pa | φ = 0.5969 rad | tan φ = 0.6796 | MC yield, SSR sweep |
| Underclay (CD) | c = 10.1 kPa | φ = 19.2° | × 10³ ; × π/180 | c = 1.01 × 10⁴ Pa | φ = 0.3351 rad | tan φ = 0.3482 | MC yield, SSR sweep |
| Fault gouge (CD) | c = 16.3 kPa | φ = 29.4° | × 10³ ; × π/180 | c = 1.63 × 10⁴ Pa | φ = 0.5131 rad | tan φ = 0.5635 | MC yield, SSR sweep |
| Slope debris (CD) | c = 8.9 kPa | φ = 21.7° | × 10³ ; × π/180 | c = 8.90 × 10³ Pa | φ = 0.3787 rad | tan φ = 0.3979 | MC yield, SSR sweep |
| B.2 RESIDUAL shear strength (design-governing values; Paper Table 3a) |  |  |  |  |  |  |  |
| Bedding, no infill — marl (water-filled box) | c = 0.0 kPa | φ = 14.4° | × 10³ ; × π/180 | c = 0 Pa | φ = 0.2513 rad | tan φ = 0.2568 | MC-SSR residual case |
| Bedding, no infill — marl (lubricated) ★ | c = 4.4 kPa | φ = 15.4° | × 10³ ; × π/180 | c = 4.40 × 10³ Pa | φ = 0.2688 rad | tan φ = 0.2754 | ★ Design pair for SSR (validated by back-analysis) |
| Bedding, slickensided w/ infill — marl | c = 0.0 kPa | φ = 12.5° | × 10³ ; × π/180 | c = 0 Pa | φ = 0.2182 rad | tan φ = 0.2217 | MC-SSR residual case |
| Bedding — tuffite | c = 0.0 kPa | φ = 32.9° | × 10³ ; × π/180 | c = 0 Pa | φ = 0.5742 rad | tan φ = 0.6469 | MC-SSR residual case |
| Underclay (CD) | c = 13.6 kPa | φ = 13.1° | × 10³ ; × π/180 | c = 1.36 × 10⁴ Pa | φ = 0.2287 rad | tan φ = 0.2327 | MC-SSR residual case |
| Fault gouge (CD) | c = 14.0 kPa | φ = 19.1° | × 10³ ; × π/180 | c = 1.40 × 10⁴ Pa | φ = 0.3334 rad | tan φ = 0.3463 | MC-SSR residual case |
| Slope debris (CD) | c = 8.0 kPa | φ = 15.8° | × 10³ ; × π/180 | c = 8.00 × 10³ Pa | φ = 0.2758 rad | tan φ = 0.2830 | MC-SSR residual case |
| B.3 Back-analysis validated parameters (limit equilibrium at F = 1; Paper Section 7) |  |  |  |  |  |  |  |
| Slickensided bedding (INS-1) | c ≈ 1.0 kPa | φ ≈ 15.0° | × 10³ ; × π/180 | c ≈ 1.0 × 10³ Pa | φ = 0.2618 rad | tan φ = 0.2679 | Validation target |
| Smooth bedding (INS-2 & INS-3) ★ | c = 4.4 kPa | φ = 15.4° | × 10³ ; × π/180 | c = 4.40 × 10³ Pa | φ = 0.2688 rad | tan φ = 0.2754 | ★ Design pair; SSR reference |

# Table C — Hoek–Brown Failure-Criterion Inputs

Source: Ulusay et al. (2014), Table 3b. Only σ_ci carries units; GSI, mᵢ, D are dimensionless.

| Rock type | Original value | Orig. unit | Conversion factor | SI value | Notes / PINN usage |
| --- | --- | --- | --- | --- | --- |
| C.1 Intact-rock UCS σ_ci: MPa → Pa (× 10⁶) [Paper Table 3b] |  |  |  |  |  |
| Sekköy Fm. (marl / marl–limestone) | σ_ci = 17.9 | MPa | × 10⁶ | σ_ci = 1.79 × 10⁷ Pa | GHB envelope; also E_rm correlations |
| Sekköy Fm. (weak zone, northern slope) | σ_ci = 4.29 | MPa | × 10⁶ | σ_ci = 4.29 × 10⁶ Pa | GHB envelope (weak zone) |
| Coal seam | σ_ci = 7.80 | MPa | × 10⁶ | σ_ci = 7.80 × 10⁶ Pa | GHB envelope (coal layer) |
| Tuffite (Yatağan Fm.) | σ_ci = 3.59 | MPa | × 10⁶ | σ_ci = 3.59 × 10⁶ Pa | GHB envelope (tuffite layer) |
| C.2 Dimensionless Hoek–Brown inputs (no conversion required) |  |  |  |  |  |
| Sekköy Fm. (marl / marl–limestone) | GSI = 50; mᵢ = 4; D = 1 | – | none | GSI = 50; mᵢ = 4; D = 1 | Feed Step 1.5 (m_b, s, a derivation) |
| Sekköy Fm. (weak zone) | GSI = 45; mᵢ = 3; D = 1 | – | none | GSI = 45; mᵢ = 3; D = 1 | Feed Step 1.5 |
| Coal seam | GSI = 40*; mᵢ = 18; D = 1 | – | none | GSI = 40; mᵢ = 18; D = 1 | *GSI from RMR (GSI = RMR₈₉ − 5) |
| Tuffite (Yatağan Fm.) | GSI = 26*; mᵢ = 15; D = 1 | – | none | GSI = 26; mᵢ = 15; D = 1 | *GSI from RMR (GSI = RMR₈₉ − 5) |

# Table D — Hydraulic & Groundwater Parameters

Source: Ulusay et al. (2014), Sections 5.2–5.5 (423 Packer/Lugeon tests, 23 monitoring wells).

| Hydrostratigraphic unit / quantity | Original value | Orig. unit | Conversion factor | SI value | Notes / PINN usage |
| --- | --- | --- | --- | --- | --- |
| D.1 Saturated hydraulic conductivity K: m/day → m/s (÷ 86 400) [Paper Section 5.2; 423 Packer/Lugeon tests] |  |  |  |  |  |
| Karstic carbonate bedrock aquifer | K ≈ 0.27 (≤ 25 Lu) | m/day | ÷ 86 400 | K ≈ 3.13 × 10⁻⁶ m/s | Lower boundary; head stays below pit floor |
| Coal seam (confined aquifer) | K = 0.1 – 0.5 (~5 Lu) | m/day | ÷ 86 400 | K = 1.16 × 10⁻⁶ – 5.79 × 10⁻⁶ m/s | Richards eq. K_s (critical aquifer) |
| Lacustrine limestone (unconfined) | K = 0.056 – 0.86 (5–70 Lu) | m/day | ÷ 86 400 | K = 6.48 × 10⁻⁷ – 9.95 × 10⁻⁶ m/s | Upper aquifer, limited extent |
| Marl, medium–thick bedded (Sekköy Fm.) | ≈ 0 (no water intake, ~0 Lu) | m/day | assigned | K ≤ 1 × 10⁻⁹ m/s (assumed aquitard value) | Impervious confining layer; value assumed for numerics |
| Underclay (Turgut Fm.) | impervious | m/day | assigned | K → 0 (model as ≤ 1 × 10⁻⁹ m/s) | Confining layer beneath coal |
| Alluvium (western boundary) | not tested (stripped) | — | — | — | Excluded from model |
| D.2 Storage and transient-flow quantities |  |  |  |  |  |
| Coal seam storage coefficient S | 3 × 10⁻⁵ (Lohman, 1972) | – | none | 3 × 10⁻⁵ (dimensionless) | Transient confined-flow term |
| Initial inflow rate (coal seam) | 0.056 | m³/day per m | ÷ 86 400 | 6.48 × 10⁻⁷ m³/s per m | Seepage-face flux BC check |
| Cumulative inflow over 30 days | 5.37 | m³ per m | none (already SI) | 5.37 m³ per m over 2.592 × 10⁶ s | Mass-balance validation |
| D.3 Hydraulic heads (already in SI metres — no conversion) |  |  |  |  |  |
| Karstic bedrock aquifer head | 113 – 197 | m asl | × 1 | 113 – 197 m | Lower Dirichlet BC |
| Coal seam head | > 205 (compartmentalised) | m asl | × 1 | > 205 m | L_IC piezometric initial condition |
| Lacustrine limestone saturated thickness | ≈ 5 | m | × 1 | ≈ 5 m | Upper aquifer geometry |

# Table E — Time & Geometry Quantities

| Quantity | Original value | Orig. unit | Conversion factor | SI value | Notes / PINN usage |
| --- | --- | --- | --- | --- | --- |
| E.1 Time quantities: day → s (× 86 400) |  |  |  |  |  |
| Analytical drawdown resolution | 1 | day | × 86 400 | 8.64 × 10⁴ s | T_ref for non-dimensionalisation (Step 1.6) |
| Significant drawdown window (~100 m reach) | 1 (24 h) | day | × 86 400 | 8.64 × 10⁴ s | Transient seepage validation |
| Inflow simulation horizon | 30 | days | × 86 400 | 2.592 × 10⁶ s | Transient run length |
| E.2 Geometry and slope angles (m already SI; angles ° → rad) |  |  |  |  |  |
| Planned pit length | 2 500 | m | × 1 | 2 500 m | Domain extent |
| Planned pit width | 750 | m | × 1 | 750 m | Domain extent |
| Maximum pit depth | 170 | m | × 1 | 170 m | L_ref for non-dimensionalisation |
| Initial overall pit-wall angle | 30 | ° | × π/180 | 0.5236 rad | Slope geometry |
| Flattened design angle (option 1) | 25 | ° | × π/180 | 0.4363 rad | Slope geometry |
| Flattened design angle (option 2) | 22 | ° | × π/180 | 0.3840 rad | Slope geometry |