Step 1.2 & 1.3 — SWCC and Elastic Parameters

Master Derivation Sheet for the Işıkdere Lignite Open-Pit Mine (Primary source: Ulusay, Ekmekçi, Tuncay & Hasançebi, 2014, Engineering Geology 181, 261–280)

Purpose: Close the two constitutive gaps left after Step 1.1. Step 1.2 derives the van Genuchten soil-water characteristic curve parameters (θr, θs, α, n, m) for every stratum, using the Rosetta3 pedotransfer model where grain-size and density data exist and documented literature/analog assignments where they do not. Step 1.3 derives the rock-mass deformation modulus E_rm via the full Hoek–Diederichs (2006) relation and assigns Poisson's ratio ν with published cross-checks. Every value carries its derivation method, its citation, and — where the method supplies one — its uncertainty, so the parameter table is defensible at review. Values marked Tier A are computed from site data; Tier B values are literature/analog assignments that must be swept in the Step 6.2 sensitivity analysis.

# Part 1 — Step 1.2: van Genuchten SWCC Parameters

## 1.1  Method

Pedotransfer model: Rosetta3 (Zhang & Schaap, 2017), executed with the USDA-ARS reference implementation, Python package rosetta-soil v0.3.2 (Python 3.12). Inputs are sand/silt/clay fractions and bulk density from Table 2 of the source paper; the package auto-selects the best-available model for each record (model 3 = SSC + bulk density; model 2 = SSC only). Reported α, n and K_sat are means of the log10-transformed bootstrap ensemble (estimate_type = "log"), the standard Rosetta convention; θr and θs are arithmetic means. Bootstrap standard deviations are reported as the parameter uncertainty and feed the Step 6.2 sensitivity ranges.

Bulk-density convention (methodological note reviewers will probe): Rosetta is trained on oven-dry bulk density, whereas Ulusay et al. report bulk (moist) unit weight γ. The correct input is therefore γ_d = γ/(1+w), converted as ρ_d = γ_d·1000/g with g = 9.81 m/s² (Step 1.1 convention). Underclay: γ_d = 18.4/1.215 = 15.14 kN/m³ → 1.544 g/cm³. Slope debris: γ_d = 17.6/1.332 = 13.21 kN/m³ → 1.347 g/cm³. The check that settles the choice: feeding the moist density (1.876 g/cm³) instead returns θs = 0.310 for the underclay — below its phase-relation porosity n₀ = 0.383, which is unphysical for a near-saturated clay — while the dry-density run returns θs = 0.391 ≈ n₀. The dry-BD runs are adopted.

## 1.2  Pedotransfer inputs (Ulusay et al., 2014, Table 2)

| Stratum | Sand / Silt / Clay (%) | γ (kN/m³) | w (–) | BD_dry (g/cm³) | USCS class | Rosetta model |
| --- | --- | --- | --- | --- | --- | --- |
| Underclay (Turgut Fm.) | 22.5 / 46.0 / 31.5 | 18.4 | 0.215 | 1.544 | CL–CH | 3 (SSC+BD) |
| Slope debris | 3.5 / 53.5 / 43.0 | 17.6 | 0.332 | 1.347 | CH | 3 (SSC+BD) |
| Fault gouge (suppl.) | 20 / 80 / 0 * | not reported | — | — | MH | 2 (SSC only) |

* Fault gouge: single specimen; clay fraction not reported in the paper and entered as 0. It is a discontinuity infill, not a stratum of the 2-D domain, and is carried as supplementary only. Underclay Gs = 2.50 (paper mean; the printed range "2.44–25.7" is an evident typographical slip for 2.44–2.57). Slope-debris Gs is not reported; 2.65 assumed for the porosity cross-check.

## 1.3  Tier A — Rosetta3-derived parameters (computed, with uncertainty)

| Stratum | θr (–) | θs (–) | α (1/m) | n (–) | m = 1−1/n | K_sat (m/s) | 1σ (log10 α / n / K_sat) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Underclay (Turgut Fm.) | 0.105 ± 0.013 | 0.391 ± 0.009 | 0.611 | 1.345 | 0.256 | 5.6 × 10⁻⁷ | 0.081 / 0.015 / 0.106 |
| Slope debris | 0.124 ± 0.017 | 0.473 ± 0.018 | 0.523 | 1.360 | 0.265 | 1.2 × 10⁻⁶ | 0.122 / 0.018 / 0.149 |
| Fault gouge (suppl.) | 0.052 | 0.445 | 0.338 | 1.706 | 0.414 | 1.2 × 10⁻⁵ | SSC-only run |

Consistency checks. (i) θs vs. phase-relation porosity: underclay 0.391 vs. n₀ = 0.383 and slope debris 0.473 vs. n₀ = 0.492 — agreement within 2–4 %, strong internal validation between the Rosetta prediction and the Step 1.1/1.5 phase relations. (ii) K_sat plausibility: Rosetta places the underclay at 5.6 × 10⁻⁷ m/s, i.e. 2–10× below the packer-tested coal seam (1.16–5.79 × 10⁻⁶ m/s), consistent with its field role as the basal aquitard. The K_sat adopted for the model remains the site-measured / Step 1.1-assigned value per stratum; the Rosetta K_sat is a plausibility check only, because a pedotransfer estimate never overrides 423 packer tests. (iii) Versus the roadmap placeholders (underclay 0.07/0.43/1.0/1.23): the computed run gives higher θr and n and lower α — the expected direction for a silt-dominated CL–CH at this density — and the computed values supersede the placeholders, which the roadmap itself labels "estimated".

## 1.4  Tier B — literature / analog assignments (no grain-size data in paper)

| Stratum | θr (–) | θs (–) | α (1/m) | n (–) | Basis and citation |
| --- | --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 0.05 | 0.38 | 0.5 | 1.20 | α = Carsel & Parrish (1988) silty-clay class value (0.005 1/cm). θs = 0.9·n₀ with n₀ = 0.427 (Step 1.5 phase relation, Gs = 2.65 assumed). n raised from the Carsel–Parrish 1.09 to 1.20 for automatic-differentiation smoothness; 1.09 kept as the sensitivity lower bound. |
| Weak-zone marl (Sekköy Fm.) | 0.05 | 0.32 | 0.5 | 1.20 | Same lithological family as the marl (identical α, n to avoid unconstrained extra degrees of freedom); θs = 0.9·n₀ from its own density (n₀ = 0.350). |
| Tuffite (Yatağan Fm.) | 0.05 | 0.37 | 1.0 | 1.30 | Analog to nonwelded pyroclastics (parameter family of Flint, 2003, Yucca Mountain matrix properties); θs = 0.9·n₀ (n₀ = 0.413, Gs = 2.50 assumed). Least-constrained SWCC in the set — mandatory Step 6.2 sweep. |
| Coal seam (lignite) | 0.02 | 0.15 | 2.0 | 1.80 | Fractured medium treated as a single-porosity equivalent continuum (concept: Peters & Klavetter, 1988; Wang & Narasimhan, 1985): θs = fracture porosity ≈ 0.15 (RQD ≈ 70 %), high α (low air entry) and high n, characteristic of fracture-dominated retention. |

The θs = 0.9·n₀ allowance reflects entrapped air and dead-end porosity: field-saturated water content typically sits 5–15 % below total porosity (Fredlund & Rahardjo, 1993). Total porosity n₀ (not θs) remains the state variable of the Kozeny–Carman feedback in Step 1.5.

## 1.5  ADOPTED master SWCC table — feeds K(ψ), C(ψ) in Steps 1.6 / 3.2 / 3.3

| Stratum | Tier | θr (–) | θs (–) | α (1/m) | α (1/Pa) | n (–) | m = 1−1/n | SWCC source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | B | 0.05 | 0.38 | 0.5 | 5.10 × 10⁻⁵ | 1.20 | 0.167 | Carsel & Parrish class + porosity tie |
| Weak-zone marl | B | 0.05 | 0.32 | 0.5 | 5.10 × 10⁻⁵ | 1.20 | 0.167 | marl analog, own porosity |
| Tuffite (Yatağan Fm.) | B | 0.05 | 0.37 | 1.0 | 1.02 × 10⁻⁴ | 1.30 | 0.231 | nonwelded-tuff analog |
| Coal seam (lignite) | B | 0.02 | 0.15 | 2.0 | 2.04 × 10⁻⁴ | 1.80 | 0.444 | fracture continuum |
| Underclay (Turgut Fm.) | A | 0.105 | 0.391 | 0.611 | 6.23 × 10⁻⁵ | 1.345 | 0.256 | Rosetta3, model 3 |
| Slope debris | A | 0.124 | 0.473 | 0.523 | 5.33 × 10⁻⁵ | 1.360 | 0.265 | Rosetta3, model 3 |
| Fault gouge (suppl.) | A | 0.052 | 0.445 | 0.338 | 3.45 × 10⁻⁵ | 1.706 | 0.414 | Rosetta3, model 2 |

α is tabulated both per metre of pressure head (for the head form of Richards' equation) and per Pascal (α/ρw·g, ρw·g = 9 810 N/m³) for the pressure form. Sensitivity flags for Step 6.2: n is the most influential parameter (a marl n of 1.20 → 1.30 shifts the wetting-front arrival at the coal seam by days); sweep marl n over 1.09–1.30, coal (α, n) over ±50 %, and the tuffite set over its full analog family. Tier A parameters carry the tabulated 1σ ranges.

# Part 2 — Step 1.3: Elastic Properties (E_rm, ν)

## 2.1  Method

Rock-mass modulus: full Hoek–Diederichs (2006) relation E_rm = E_i · [0.02 + (1 − D/2) / (1 + exp((60 + 15D − GSI)/11))], with intact modulus estimated as E_i = MR · σ_ci, where MR is the modulus ratio of Deere (1968) as tabulated by Hoek & Diederichs (2006, after Palmström & Singh, 2001). Disturbance factor D = 1 throughout: open-pit slopes with production blasting (Hoek et al., 2002 guidance). Using the intact modulus directly would be wrong for slope work — the mass deforms on its joints, and at these GSI values the Hoek–Diederichs reduction factor is only 0.026–0.067 of E_i. The simplified Hoek–Diederichs equation (GSI and D only, no σ_ci) is computed as a cross-check but not adopted: because it ignores the exceptionally low intact strengths at this site (σ_ci = 3.6–17.9 MPa), it overestimates E_rm by roughly 2–9× here.

## 2.2  Rock-mass modulus derivation (D = 1)

| Stratum | GSI | σ_ci (MPa) | MR range → adopted | E_i (MPa) | HD factor | E_rm (MPa) | E_rm range (MPa) | Simplified-HD (MPa) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 50 | 17.9 | 150–200 → 175 | 3 133 | 0.0667 | 209 | 179 – 239 | 525 |
| Weak-zone marl | 45 | 4.29 | 150–200 → 175 | 751 | 0.0507 | 38.1 | 32.6 – 43.5 | 335 |
| Coal seam (lignite) | 40 | 7.80 | 200–400 → 300 * | 2 340 | 0.0399 | 93.4 | 62.3 – 124.6 | 213 |
| Tuffite (Yatağan Fm.) | 26 | 3.59 | 200–400 → 350 | 1 257 | 0.0257 | 32.3 | 18.5 – 37.0 | 60 |

* Coal is absent from the Hoek–Diederichs MR table; MR = 300 is adopted (roadmap value, consistent with lignite intact moduli of ≈ 1–3 GPa at σ_ci ≈ 8 MPa) and bracketed 200–400 for sensitivity. Marl and tuffite MR ranges are read directly from the Hoek–Diederichs (2006) table: Marls 150–200; Tuff 200–400.

Correction against the roadmap draft (record this — reviewers will ask): the roadmap's worked example used MR = 400 for the marl, giving E_rm ≈ 478 MPa. The published Hoek–Diederichs table assigns Marls MR = 150–200, i.e. the draft value lies at twice the tabulated upper bound. This sheet adopts the mid-range MR = 175 → E_rm ≈ 209 MPa and carries 478 MPa only as an upper sensitivity case. Coal (93.4 vs 94.5 MPa) and tuffite (32.3 vs 32.4 MPa) reproduce the roadmap arithmetic to rounding. The large simplified-vs-full gap for the weak-zone marl (335 vs 38 MPa) is expected: its σ_ci of 4.29 MPa is far weaker than its GSI of 45 alone would suggest, which is precisely the information the simplified form discards.

## 2.3  Poisson's ratio — rock strata

| Stratum | ν adopted | Basis | Vásárhelyi (2009) rock-mass estimate | Note |
| --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 0.28 | typical marl / mudstone range (Gerçek, 2007) | 0.345 | rock-mass est. = upper bound |
| Weak-zone marl | 0.30 | as marl, softened zone | 0.358 | rock-mass est. = upper bound |
| Coal seam (lignite) | 0.32 | typical lignite / coal range (Gerçek, 2007) | 0.323 | two methods agree |
| Tuffite (Yatağan Fm.) | 0.33 | weak pyroclastics (Gerçek, 2007) | 0.360 | rock-mass est. = upper bound |

Cross-check relation: ν_rm = −0.002·GSI − 0.003·m_i + 0.457 (Vásárhelyi, 2009), which raises ν as mass quality degrades. Adopted values are the intact-typical figures; the Vásárhelyi estimates define the upper end of the Step 6.2 sweep. ν is second-order for the factor of safety but first-order for the initial lateral stress ratio K₀ ≈ ν/(1−ν), so the sweep matters for the geostatic initial condition.

## 2.4  Soil strata — drained E′, ν′ (weakest-constrained values in the sheet)

| Stratum | E′ (MPa), adopted (range) | ν′ (–) | Basis |
| --- | --- | --- | --- |
| Underclay (Turgut Fm.) | 15 (10 – 25) | 0.35 | Stiff CL–CH clay at w = 21.5 %; typical drained-modulus and ν′ ranges for stiff high-plasticity clays (Kulhawy & Mayne, 1990; Bowles, 1996). No site oedometer/triaxial stiffness data exist. |
| Slope debris | 8 (5 – 15) | 0.35 | Firm–stiff CH colluvium at w = 33.2 % (liquidity index ≈ 0.14); same sources. |

Drained (effective-stress) constants are the correct choice because the coupled formulation carries pore pressure explicitly through σ′ = σ − u_w·I; the undrained limit ν → 0.5 must not be used, and would in any case make the plane-strain elasticity tensor singular. These two moduli are literature-typical, not site-derived — the least-constrained parameters in Steps 1.2–1.3. They are mandatory Step 6.2 sweep variables, and the clean upgrade path is a small oedometer / CD-triaxial program on underclay block samples.

## 2.5  ADOPTED elastic set in SI units — builds the plane-strain tensor D, feeds Step 1.6 scaling

| Stratum | E (Pa) | ν (–) | G = E/2(1+ν) (Pa) | λ = Eν/[(1+ν)(1−2ν)] (Pa) |
| --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 2.089 × 10⁸ | 0.28 | 8.162 × 10⁷ | 1.039 × 10⁸ |
| Weak-zone marl | 3.807 × 10⁷ | 0.30 | 1.464 × 10⁷ | 2.196 × 10⁷ |
| Coal seam (lignite) | 9.344 × 10⁷ | 0.32 | 3.539 × 10⁷ | 6.292 × 10⁷ |
| Tuffite (Yatağan Fm.) | 3.235 × 10⁷ | 0.33 | 1.216 × 10⁷ | 2.361 × 10⁷ |
| Underclay (Turgut Fm.) | 1.500 × 10⁷ | 0.35 | 5.556 × 10⁶ | 1.296 × 10⁷ |
| Slope debris | 8.000 × 10⁶ | 0.35 | 2.963 × 10⁶ | 6.914 × 10⁶ |

The stiffness contrast across strata spans a factor of ≈ 26 (slope debris → marl). Record E_ref ≈ 10⁸ Pa as the candidate stress/stiffness scale for the Step 1.6 non-dimensionalisation so all mechanical residuals sit at O(1).

# Part 3 — Derivation-method and citation register

One line per parameter group: what was done, and the citation that backs it. This is the table to hand a reviewer.

| Parameter set | Method | Citation(s) |
| --- | --- | --- |
| SWCC — underclay, slope debris, (gouge) | Rosetta3 pedotransfer, models 3/2, log-ensemble estimates, dry-BD inputs | Zhang & Schaap (2017); Schaap, Leij & van Genuchten (2001); rosetta-soil v0.3.2 (USDA-ARS) |
| SWCC — marl, weak-zone marl | Texture-class value for α + porosity-tied θs; n bracketed | Carsel & Parrish (1988); Fredlund & Rahardjo (1993) |
| SWCC — tuffite | Nonwelded-pyroclastic analog, porosity-tied θs | Flint (2003); Fredlund & Rahardjo (1993) |
| SWCC — coal | Single-porosity equivalent continuum for a fractured medium; θs = fracture porosity | Peters & Klavetter (1988); Wang & Narasimhan (1985) |
| SWCC functional form | van Genuchten–Mualem, m = 1 − 1/n (C∞-smooth under autodiff) | van Genuchten (1980); Mualem (1976) |
| E_rm — all rock strata | Full Hoek–Diederichs with E_i = MR·σ_ci, site GSI, D = 1; simplified form as cross-check only | Hoek & Diederichs (2006); Deere (1968); Palmström & Singh (2001); Hoek, Carranza-Torres & Corkum (2002) |
| ν — rock strata | Intact-typical adoption + rock-mass estimate as upper bound | Gerçek (2007); Vásárhelyi (2009) |
| E′, ν′ — soil strata | Literature-typical drained values for stiff high-plasticity clays | Kulhawy & Mayne (1990); Bowles (1996) |
| Site inputs (SSC, γ, w, Gs, GSI, σ_ci, m_i) | Direct extraction, Step 1.1 SI conversions | Ulusay et al. (2014), Tables 2 & Hoek–Brown inputs |

# Part 4 — Verification and reproducibility

Hand checks (Day-8 protocol, three parameters re-derived independently):

(1) Marl E_rm: E_i = 175 × 17.9 = 3 132.5 MPa; exponent (60 + 15 − 50)/11 = 2.2727, exp = 9.706; factor = 0.02 + 0.5/10.706 = 0.0667; E_rm = 3 132.5 × 0.0667 = 208.9 MPa ✓.

(2) Underclay θs vs porosity: γ_d = 18.4/1.215 = 15.14 kN/m³; ρ_d = 1 543.7 kg/m³; n₀ = 1 − 1.544/2.50 = 0.383; Rosetta θs = 0.391, difference 2 % ✓.

(3) Tuffite HD factor: (75 − 26)/11 = 4.4545, exp = 86.05; factor = 0.02 + 0.5/87.05 = 0.02574; E_rm = 1 256.5 × 0.02574 = 32.3 MPa ✓.

Reproducibility: derive_step_1_2_1_3.py (this sheet's companion script), Python 3.12, rosetta-soil 0.3.2, rosetta_version = 3, estimate_type = "log"; machine-readable adopted set in step_1_2_1_3_adopted_parameters.csv / .json for direct ingestion by the PINN code. Commit all three to the repository alongside this sheet.

# References

Bowles, J. E. (1996). Foundation Analysis and Design (5th ed.). McGraw-Hill.

Carsel, R. F., & Parrish, R. S. (1988). Developing joint probability distributions of soil water retention characteristics. Water Resources Research, 24(5), 755–769.

Deere, D. U. (1968). Geological considerations. In K. G. Stagg & O. C. Zienkiewicz (Eds.), Rock Mechanics in Engineering Practice (pp. 1–20). Wiley.

Flint, L. E. (2003). Physical and hydraulic properties of volcanic rocks from Yucca Mountain, Nevada. Water Resources Research, 39(5), 1119.

Fredlund, D. G., & Rahardjo, H. (1993). Soil Mechanics for Unsaturated Soils. Wiley.

Gerçek, H. (2007). Poisson's ratio values for rocks. International Journal of Rock Mechanics and Mining Sciences, 44(1), 1–13.

Hoek, E., Carranza-Torres, C., & Corkum, B. (2002). Hoek–Brown failure criterion — 2002 edition. Proc. NARMS-TAC Conference, Toronto, 267–273.

Hoek, E., & Diederichs, M. S. (2006). Empirical estimation of rock mass modulus. International Journal of Rock Mechanics and Mining Sciences, 43(2), 203–215.

Kulhawy, F. H., & Mayne, P. W. (1990). Manual on Estimating Soil Properties for Foundation Design. EPRI Report EL-6800.

Mualem, Y. (1976). A new model for predicting the hydraulic conductivity of unsaturated porous media. Water Resources Research, 12(3), 513–522.

Palmström, A., & Singh, R. (2001). The deformation modulus of rock masses — comparisons between in situ tests and indirect estimates. Tunnelling and Underground Space Technology, 16(2), 115–131.

Peters, R. R., & Klavetter, E. A. (1988). A continuum model for water movement in an unsaturated fractured rock mass. Water Resources Research, 24(3), 416–430.

Schaap, M. G., Leij, F. J., & van Genuchten, M. Th. (2001). ROSETTA: a computer program for estimating soil hydraulic parameters with hierarchical pedotransfer functions. Journal of Hydrology, 251(3–4), 163–176.

Ulusay, R., Ekmekçi, M., Tuncay, E., & Hasançebi, N. (2014). Improvement of slope stability based on integrated geotechnical evaluations and hydrogeological conceptualisation at a lignite open pit. Engineering Geology, 181, 261–280.

van Genuchten, M. Th. (1980). A closed-form equation for predicting the hydraulic conductivity of unsaturated soils. Soil Science Society of America Journal, 44(5), 892–898.

Vásárhelyi, B. (2009). A possible method for estimating the Poisson's rate values of the rock masses. Acta Geodaetica et Geophysica Hungarica, 44(3), 313–322.

Wang, J. S. Y., & Narasimhan, T. N. (1985). Hydrologic mechanisms governing fluid flow in a partially saturated, fractured, porous medium. Water Resources Research, 21(12), 1861–1874.

Zhang, Y., & Schaap, M. G. (2017). Weighted recalibration of the Rosetta pedotransfer model with improved estimates of hydraulic parameter distributions and summary statistics (Rosetta3). Journal of Hydrology, 547, 39–53.