Step 1.5 — Hoek–Brown Rock-Mass Parameters and Mohr–Coulomb Equivalents

Işıkdere open-pit lignite mine — PINN slope-stability thesis

# 1. Objective

Derive the Generalized Hoek–Brown (GHB) reduced parameters m_b, s, and a for each rock unit at the Işıkdere pit from GSI, σ_ci, m_i, and the disturbance factor D; cross-check the derivation against the values implied in the source paper; and compute the equivalent Mohr–Coulomb (MC) cohesion c and friction angle φ that the later MC-vs-GHB SSR comparison requires.

# 2. Governing equations (Hoek et al., 2002)

m_b = m_i · exp[(GSI − 100) / (28 − 14D)]

s = exp[(GSI − 100) / (9 − 3D)]

a = ½ + (1/6)·[exp(−GSI/15) − exp(−20/3)]

D = 1 for every unit — production blasting and stress relief in a large open pit (Ulusay et al., 2014). GSI, σ_ci and m_i are taken directly from the paper's Table 3b; GSI for coal and tuffite is derived there from RMR (GSI = RMR₈₉ − 5).

# 3. Reduced Hoek–Brown parameters

| Rock unit | GSI | σ_ci (MPa) | m_i | m_b | s | a |
| --- | --- | --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 50 | 17.9 | 4 | 0.1125 | 2.40×10⁻⁴ | 0.5057 |
| Weak zone (marl, N slope) | 45 | 4.29 | 3 | 0.0590 | 1.05×10⁻⁴ | 0.5081 |
| Coal seam | 40 | 7.80 | 18 | 0.2477 | 4.54×10⁻⁵ | 0.5114 |
| Tuffite (Yatağan Fm.) | 26 | 3.59 | 15 | 0.0760 | 4.40×10⁻⁶ | 0.5292 |

Cross-check. The paper states that m_b, s and a were assessed with the 2002 criterion from exactly these GSI/σ_ci/m_i/D inputs, but does not print the reduced values. An independent recomputation (script provided) reproduces the project-roadmap values to all shown digits, confirming the derivation is internally consistent. The expected magnitudes hold: m_b ≈ 0.06–0.25 for GSI in the 26–50 band, s ≪ 10⁻³ (heavily fractured, D = 1), and a clustering just above 0.51.

# 4. Mohr–Coulomb equivalents (design confining range)

Fitted with the Hoek (2002) closed-form MC linearization over 0 ≤ σ′₃ ≤ σ′₃max, using σ′₃max ≈ 0.25·γH (γ = 20 kN/m³, H ≈ 170 m ⇒ σ′₃max ≈ 850 kPa).

| Rock unit | c (kPa) | φ (°) | from m_b / s / a |
| --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 130.5 | 21.7 | 0.1125 / 2.40e-4 / 0.506 |
| Weak zone (marl) | 50.4 | 10.0 | 0.0590 / 1.05e-4 / 0.508 |
| Coal seam | 108.2 | 21.6 | 0.2477 / 4.54e-5 / 0.511 |
| Tuffite (Yatağan Fm.) | 43.9 | 9.9 | 0.0760 / 4.40e-6 / 0.529 |

# 5. Sensitivity of the MC fit to σ′₃max

The equivalent c and φ are not material constants — they depend entirely on the confining range fitted. The table below (c in kPa / φ in degrees) shows how strongly they shift; near a free face (σ′₃ → 0) MC systematically over-predicts strength relative to the curved GHB envelope. Choose σ′₃max deliberately for the SSR runs and state it explicitly.

| Rock unit | 25 kPa | 100 kPa | 400 kPa | 850 kPa |
| --- | --- | --- | --- | --- |
| Marl (Sekköy Fm.) | 55.3 / 43.2 | 61.8 / 36.6 | 92.4 / 27.0 | 130.5 / 21.7 |
| Weak zone (marl) | 10.8 / 30.9 | 16.8 / 21.7 | 33.4 / 13.5 | 50.4 / 10.0 |
| Coal seam | 11.6 / 49.6 | 26.9 / 38.5 | 66.6 / 27.2 | 108.2 / 21.6 |
| Tuffite (Yatağan Fm.) | 4.7 / 31.3 | 11.6 / 21.2 | 27.8 / 13.2 | 43.9 / 9.9 |

# 6. Important caveat for the comparison

The GHB-derived MC pairs above describe the intact rock-mass envelope. They are far stronger than the back-analysis-validated discontinuity strengths (c = 4.4 kPa, φ = 15.4°; residual, smooth bedding) that govern the actual bedding-controlled failures at Işıkdere. These are two different mechanisms. The MC-SSR sweep (Step 5.1) uses the discontinuity / back-analysis values; the GHB-SSR sweep (Step 5.2) uses the m_b, s, a in Table 1. The MC-equivalents here exist only to show what an MC idealization of the rock mass would give versus the true GHB envelope — that contrast is part of the comparison's contribution.

# 7. Provenance of inputs

GSI, σ_ci, m_i, D: Ulusay, Ekmekçi et al. (2014), Engineering Geology 181, Table 3b — as recorded in Isikdere_Mine_Extracted_Data.docx (Table 3). Reduction and MC-fit equations: Hoek, Carranza-Torres & Corkum (2002). All values reproducible via step1_5_hoek_brown.py.