Physics-Informed Neural Networks for

Coupled Hydro-Mechanical Slope Stability Analysis

in Open-Pit Mines

Comprehensive Equations, Data Requirements & Field Data Acquisition Guide

Coupling Strategy: Two-Way Monolithic (Full Poroelastic Feedback)

Failure Criteria: Mohr-Coulomb + Generalized Hoek-Brown (Comparison Study)

Data Source: Field Data Acquisition from Open-Pit Mine

Target: Q1 Journal Publication

# Table 1: Complete Governing Equations

All 22 equations and relations required for the monolithic two-way coupled PINN framework with dual failure criteria (Mohr-Coulomb + Generalized Hoek-Brown). Organized across 5 physical domains.

| Equation Name | Mathematical Expression | Physical Purpose | Role in PINN | Priority |
| --- | --- | --- | --- | --- |
| SEEPAGE DOMAIN — Richards Equation |  |  |  |  |
| Mixed-Form Richards Equation | ∂θ/∂t = ∇·[K(ψ)∇(ψ + z)] + S | Governs transient moisture transport through unsaturated porous media | Core PDE residual loss (L_PDE) — seepage component | Essential |
| Pressure-Head Form (Expanded) | C(ψ)·∂ψ/∂t − ∂K/∂ψ·(∂ψ/∂x)² − K(ψ)·∂²ψ/∂x² − ∂K/∂ψ·∂ψ/∂x = 0 | Explicit differential form written directly into the PINN loss function | Enables AD-based gradient computation; isolates exact derivatives of ψ | Essential |
| Mualem–van Genuchten SWCC | Θ = (θ − θ_r)/(θ_s − θ_r) = [1 + (α¦ψ¦)ⁿ]⁻ᵐ, ψ < 0 | Constitutive model linking pressure head to volumetric water content | Must be continuously differentiable for AD backpropagation | Essential |
| Hydraulic Conductivity Function | K(ψ) = K_s·Θ½·[1 − (1 − Θ^(1/m))^m]² | Relates unsaturated conductivity to effective saturation | Embedded in Richards PDE residual; extreme nonlinearity source | Essential |
| Specific Moisture Capacity | C(ψ) = ∂θ/∂ψ | Rate of change of water content w.r.t. pressure head | Critical for chain-rule expansion in backpropagation | Essential |
| VG Parameter Constraint | m = 1 − 1/n | Reduces free parameters in van Genuchten model | Reduces hyperparameter search space | Essential |
| MECHANICAL DOMAIN — Equilibrium & Constitutive |  |  |  |  |
| Linear Momentum Balance | ∇·σ + ρ_b·g = 0 | Global mechanical equilibrium of the porous medium | Core PDE residual loss (L_PDE) — mechanical component | Essential |
| Terzaghi Effective Stress | σ′ = σ − u_w·I | Partitions total stress below water table | Couples seepage output (u_w) to mechanical solver | Essential |
| Bishop Generalized Effective Stress | σ′ = σ − u_a·I + χ(u_a − u_w)·I | Accounts for matric suction in unsaturated zone; χ ≈ Θ | Direct two-way coupling: saturation → effective stress | Essential |
| Linear Elastic Constitutive | σ = D·ε (plane strain or 3D) | Relates stress to strain via elasticity tensor D(E, ν) | Defines stress-displacement relationship | Essential |
| Strain-Displacement | ε = ½(∇u + ∇uᵀ) — small deformation | Links displacement field to strain tensor | AD computes ∂u/∂x, ∂v/∂z directly from network output | Essential |
| Bulk Density (Saturation-Dependent) | ρ_b = ρ_d + θ·ρ_w | Body force depends on moisture content | Creates feedback loop: seepage → body forces → equilibrium | Essential |
| TWO-WAY MONOLITHIC COUPLING |  |  |  |  |
| Porosity–Strain Feedback | n = n₀ + ε_v; ε_v = ∂u/∂x + ∂v/∂z | Volumetric strain alters porosity | Feeds updated porosity back into K(ψ) and θ(ψ) | Essential |
| Kozeny–Carman Permeability Update | K_s = K_s0·[n/n₀]³·[(1−n₀)/(1−n)]² | Updates hydraulic conductivity from deformation | Closes two-way feedback loop in monolithic loss | Essential |
| Biot’s Poroelastic Storage | S_s = (α_B − n)/K_solid + n/K_fluid | Specific storage for solid and fluid compressibility | Modifies Richards equation storage term | Recommended |
| SLOPE STABILITY — SSR + FAILURE CRITERIA |  |  |  |  |
| Mohr-Coulomb Strength Reduction | c′_r = c′/SRF; φ′_r = arctan(tanφ′/SRF) | Iteratively weakens soil strength until limit equilibrium | Parametric sweep: retrain PINN at each SRF increment | Essential |
| Mohr-Coulomb Yield Criterion | τ_f = c′ + σ_n′·tan(φ′) | Defines shear strength envelope for soil/weak rock | Yield check at collocation points | Essential |
| Generalized Hoek-Brown Criterion | σ′₁ = σ′₃ + σ_ci·(m_b·σ′₃/σ_ci + s)^a | Nonlinear strength envelope for rock masses | GHB parameters reduced by SRF for rock slope SSR | Essential |
| Hoek-Brown to Mohr-Coulomb Fit | Fitted c′ and φ′ from GHB over σ′₃ range | Enables direct FOS comparison between both criteria | Validates consistency between MC and GHB SSR | Essential |
| Factor of Safety Definition | FOS = SRF_critical | Quantifies stability; FOS > 1.0 = stable | Detected when L_PDE flatlines or ∇u → ∞ | Essential |
| LOSS FUNCTION & TRAINING |  |  |  |  |
| Total PINN Loss (Monolithic) | L = w_PDE·L_PDE + w_BC·L_BC + w_IC·L_IC + w_data·L_data | Multi-objective optimization target | Single unified loss for monolithic two-way coupling | Essential |
| PDE Residual Loss | L_PDE = (1/N)·Σ¦F_Richards¦² + ¦F_Mechanical¦² | Enforces both governing PDEs simultaneously | Evaluated at interior collocation points via AD | Essential |
| Non-Dimensionalization | x* = x/L_ref, t* = t/T_ref, ψ* = ψ/H_ref | Normalizes all variables to O(1) | Essential for monolithic training convergence | Essential |

# Table 2: Calculation Data Requirements

Complete inventory of 28 parameters needed to execute all calculations. Includes units, acquisition methods, equation linkages, and typical value ranges.

| Parameter | Unit | How to Obtain | Used In | Notes / Typical Values |
| --- | --- | --- | --- | --- |
| SOIL/ROCK HYDRAULIC PROPERTIES |  |  |  |  |
| Saturated hydraulic conductivity (K_s) | m/s | Lab permeability tests, slug tests, pump tests | Richards equation, HCF | Varies by orders of magnitude across strata; most sensitive parameter |
| van Genuchten α | 1/m | SWCC lab test (pressure plate, hanging column) | SWCC curve fitting | Inversely related to air-entry pressure |
| van Genuchten n | — | SWCC lab test curve fitting | SWCC, HCF, C(ψ) | Controls pore-size distribution; m = 1 − 1/n |
| Saturated water content (θ_s) | m³/m³ | Lab measurement or ≈ porosity | SWCC, effective saturation Θ | Typically 0.3–0.5 for soils; lower for rock |
| Residual water content (θ_r) | m³/m³ | SWCC lab test (asymptotic value) | SWCC, effective saturation Θ | Typically 0.02–0.10 |
| Initial porosity (n₀) | — | Lab measurement, borehole logging | Porosity–strain feedback, Kozeny–Carman | Different for each geological stratum |
| MECHANICAL STRENGTH PROPERTIES |  |  |  |  |
| Effective cohesion (c′) | kPa | Triaxial test (CD/CU), direct shear | Mohr-Coulomb yield & SSR | Reduced iteratively: c′_r = c′/SRF |
| Effective friction angle (φ′) | degrees | Triaxial test, direct shear | Mohr-Coulomb yield & SSR | Reduced: φ′_r = arctan(tanφ′/SRF) |
| Young’s modulus (E) | MPa | Triaxial test, plate load test, pressuremeter | Hooke’s law, elasticity tensor D | May vary with confining pressure and saturation |
| Poisson’s ratio (ν) | — | Triaxial test | Elasticity tensor D | Typically 0.25–0.35 (soil); 0.15–0.30 (rock) |
| Uniaxial compressive strength (σ_ci) | MPa | UCS test on intact rock cores | Generalized Hoek-Brown criterion | Intact rock strength; essential for GHB |
| Geological Strength Index (GSI) | — | Field mapping (rock structure + surface quality) | Derives m_b, s, a for GHB | Ranges 10–100; from joint conditions |
| Intact rock constant (m_i) | — | Triaxial testing or published tables (Hoek) | Derives m_b for GHB | Rock-type dependent (granite ~32, shale ~6) |
| Disturbance factor (D) | — | Engineering judgment (blasting/excavation) | GHB parameter derivation | D = 0 (undisturbed) to 1 (heavy blast damage) |
| Unit weight / Dry density (γ_d, ρ_d) | kN/m³ | Lab measurement | Body forces (ρ_b·g), geostatic stress | Needed per stratum; ρ_b = ρ_d + θ·ρ_w |
| BOUNDARY & INITIAL CONDITIONS |  |  |  |  |
| Initial pore pressure / water table | m or kPa | Piezometer readings, hydrogeological survey | Initial condition loss (L_IC) | Defines hydrostatic baseline at t = 0 |
| Rainfall intensity & duration | mm/hr | Meteorological station data, IDF curves | Neumann BC (infiltration flux) | Critical transient loading; drives wetting front |
| Fixed head / pressure at boundaries | m | Known reservoir levels, drainage outlets | Dirichlet BC for seepage | Water table elevation at pit floor / far-field |
| Displacement constraints | — | Engineering judgment (bedrock = fixed base) | Dirichlet BC for mechanics | Roller BCs on lateral boundaries |
| Free surface / excavation face stress | kPa | Excavation geometry (σ_n = 0 on free face) | Neumann BC for mechanics | Stress-free condition on excavated benches |
| PINN ARCHITECTURE & TRAINING HYPERPARAMETERS |  |  |  |  |
| Network depth & width | — | Hyperparameter tuning / literature | FNN architecture (e.g., 8 layers × 64 neurons) | Deeper networks for complex geometries |
| Activation function | — | Design choice | tanh preferred (infinite differentiability) | Avoid ReLU — not C² differentiable |
| Collocation point counts | integer | Convergence study | Sampling density for each loss term | Adaptive resampling (RAR) near wetting fronts |
| Loss weights (w_PDE, w_BC, w_IC, w_data) | — | Adaptive algorithms (GradNorm, NTK, VARI) | Balance multi-objective loss | Static weights inadequate for monolithic coupling |
| Reference scales (L_ref, T_ref, H_ref) | varies | Domain dimensions & physics | Non-dimensionalization | Must normalize all variables to O(1) |
| Adam learning rate & epochs | — | Tuning (typical: 1e-3 → 1e-4, 50k–200k) | Stage 1 optimizer | Warm-up phase; break symmetries |
| L-BFGS convergence tolerance | — | Target residual (e.g., 1e-6) | Stage 2 optimizer | Quadratic convergence near minima |
| SRF increment (ΔSRF) | — | Design choice (typical: 0.05) | SSR parameter sweep | Smaller steps → more precise FOS |

# Table 3: Open-Pit Mine Field Data Acquisition Checklist

30 data items organized by collection category. This serves as the field campaign planning document for acquiring real data from the target open-pit mine.

| Data Required from Mine | Collection Method / Instrument | Why You Need It (Connection to PINN Framework) |
| --- | --- | --- |
| GEOLOGICAL & GEOTECHNICAL SURVEY |  |  |
| Lithological logs from exploration boreholes | Core drilling, RQD logging | Defines stratum boundaries, material layering for domain decomposition |
| Geological cross-sections of pit walls | Geological mapping + interpolation | Defines slope geometry, bench configuration, and layer interfaces |
| Structural geology (faults, joints, discontinuities) | Scanline/window mapping, televiewer | Identifies potential failure planes; needed for GSI assessment |
| Rock mass classification (GSI, RMR, Q) | Field mapping per Hoek-Brown guidelines | Derives GHB parameters m_b, s, a from GSI |
| Blasting & excavation disturbance records | Blast monitoring, excavation logs | Determines Disturbance factor D for Hoek-Brown |
| Current pit geometry (3D or 2D sections) | LiDAR survey, drone photogrammetry, total station | Defines computational domain and boundary geometry |
| HYDROGEOLOGICAL DATA |  |  |
| Piezometric head readings (time-series) | Vibrating-wire piezometers, standpipe piezometers | Initial conditions (L_IC), validation data (L_data), water table position |
| Groundwater table elevation map | Piezometer network interpolation | Defines phreatic surface for Terzaghi ↔ Bishop stress transition |
| Rainfall records (intensity, duration, frequency) | On-site weather station, regional IDF curves | Surface flux boundary condition for Richards equation |
| Dewatering well locations & pumping rates | Mine operations records | Sink terms (S) in Richards equation; Neumann/Dirichlet BCs |
| In-situ permeability profiles | Packer tests, Lugeon tests, slug tests | K_s values per stratum; spatial conductivity mapping |
| Evapotranspiration data | Weather station (temp, humidity, wind) | Surface flux BC correction (outflow from slope surface) |
| LABORATORY TESTING PROGRAM |  |  |
| SWCC curves per stratum | Pressure plate apparatus, filter paper method | Fit van Genuchten parameters (α, n, θ_s, θ_r) per layer |
| Saturated permeability per stratum | Constant/falling head permeameter | K_s for each geological unit |
| Triaxial compression tests (CD or CU+u) | Consolidated drained/undrained triaxial | c′, φ′, E, ν for Mohr-Coulomb; stress-strain curves |
| UCS tests on intact rock cores | Unconfined compression test | σ_ci for Hoek-Brown criterion |
| Hoek triaxial tests (varying confinement) | Triaxial at multiple σ₃ levels | Fit m_i for intact rock; validate GHB envelope |
| Grain size distribution & Atterberg limits | Sieve analysis, hydrometer, liquid/plastic limit | Soil classification; estimate initial van Genuchten parameters |
| Specific gravity & moisture content | Pycnometer, oven drying | Calculate ρ_d, γ, porosity n₀ |
| MONITORING & VALIDATION DATA |  |  |
| Surface displacement measurements | Survey prisms, InSAR, GPS monuments | Validation data (L_data): compare PINN displacement output |
| Subsurface displacement profiles | Inclinometers, ShapeAccelArray (SAA) | Depth-wise displacement validation; detect shear band location |
| Tensiometer / moisture sensor readings | Tensiometers (shallow), TDR probes | Validate ψ(x,z,t) and θ(x,z,t) predictions from PINN |
| Historical slope failure records | Mine incident reports, aerial photos | Calibrate FOS predictions; validate failure mechanism geometry |
| Seismic / microseismic data (if available) | Geophone array | Identify active shear zones for comparison with PINN shear bands |
| MINE OPERATIONS & PLANNING |  |  |
| Excavation sequence & schedule | Mine planning department records | Time-dependent boundary changes; progressive bench removal |
| Bench geometry (height, width, face angle) | Design drawings, survey data | Defines computational domain geometry precisely |
| Overall slope angle (inter-ramp, overall) | Design specifications | Critical geometric parameter for stability analysis |
| Depressurization / dewatering schedule | Operations records, pump data | Time-varying hydraulic BCs for transient simulation |
| Blast vibration records (PPV) | Blast monitoring (seismographs) | Dynamic loading input; justifies Disturbance factor D |

Summary: This document contains 22 governing equations, 28 calculation parameters, and 30 field data items required to implement the complete PINN-based coupled hydro-mechanical slope stability framework with monolithic two-way coupling and dual failure criteria comparison (Mohr-Coulomb + Generalized Hoek-Brown) for open-pit mine applications.